# MULTIPLE CHOICE QUESTIONS (1 mark each)

What can you assume if you successfully boot a computer up into Windows, but don't hear any beep during the POST?

A. It’s not getting any power.

B. Everything is as it should be.

C. The speaker is broken.

D. POST is corrupted.

John's old 386 has only a Primary IDE controller. What is the maximum number of IDE devices he can add to that controller?

A. 1

B. 2

C. 3

D. 4

What type of bus does Firewire use?

A. Serial

B. Parallel

C. Star

D. Network

John is building a system from older parts that he has acquired. He has a socket 7 motherboard and a Pentium 100MHz. The motherboard has both SIMM and DIMM slots. He decides to use the DIMM slots. How many sticks of 168-pin DIMMs will it take to make one bank on his motherboard?

A. 1

B. 2

C. 4

D. 6

Which of the following is a benefit of flash ROM over older types of ROM?

A. You can update your system BIOS via software.

B. You can update your expansion bus type via software.

C. You can update the chipset through software.

D. You can update your drive controllers via software.

Which type of cable has 50 pins?

A. Floppy drive ribbon cable.

B. Internal narrow SCSI cable.

C. EIDE ribbon cable.

D. IDE ribbon cable.

Which CPU uses a Slot A motherboard connection?

A. Pentium

B. Pentium MMX

C. Pentium II

D. Athlon

What is the basic storage unit of a formatted hard drive?

- cylinder

- cluster

- segment

- head

A narrow SCSI chain can have up to how many devices?

A. 4

B. 7

C. 8

D. 32

The maximum speed for the ISA bus is?

A. 4.77 MHz

B. 5 MHz

C. 7 MHz

D. 8.33 MHz

Theoretically, with an address bus that is 32 bits, you can _____________

A. use Sector Translation with your hard drive

B. use 32-bit processing.

C. have up to 4 gigabytes of RAM.

D. partition your hard drive as one drive, without wasting space.

If your floppy drive cable is on backwards, _____

A. your data will be inversely read.

B. your system will register a floppy error upon booting.

C. it just doesn't matter.

D. the floppy drive light will stay on as long as your system has power, and will not read diskettes.

George wants the boot sequence of his computer to be A, C, SCSI. Where does he go to change the boot sequence setting?

- BIOS

- He cannot change it.

- My Computer

- CMOS setup

Jill installs a new SCSI host adapter in a system. When she boots up the system, a new text message appears and seems to cause the system to halt for a minute. Then the system continues, and Windows loads normally. The host adapter appears in her Device Manager and seems to be working. What could be the cause of the odd text message and slowdown at boot?

- The SCSI host adapter must have buggy drivers

- The SCSI host adapter must need drivers loaded.

- The SCSI host adapter has option ROM that loads BIOS for the card

- The SCSI host adapter has option RAM that has to search the hard drive for drivers before it can load.

What type of connector attached the AT power supply to the AT motherboard? (Select all that apply)

- P1

- P2

- P8

- P9

What do you need to remember when connecting the AT power connectors to the motherboard?

- Keep the red wires together

- Keep the black wires together

- Keep the brown wires together

- It is impossible to put the connectors in wrong

What IRQs could an 8-bit ISA card theoretically use?

- 0 – 4

- 0 – 7

- 0 – 15

- 2 – 15

What do PCI slots look like?

- Small and white

- Small and brown

- Small and green

- Large and black

What type of cards can you put in an AGP slot?

- Modem

- Video

- NIC

- PCI

If your CMOS isn’t capable of auto detecting your hard drive, which of the following values do you need to specify? (Select all that apply)

- Heads

- Cylinders

- Clusters

- Sectors

At startup, where does your PC receive its first set of instructions?

A.	Operating system

B.	RAM BIOS

C.	ROM BIOS

D.	CMOS

How do Pentium II processors connect to the motherboard?

A.	Socket 7

B.	Socket 8

C.	Slot 1

D.	Slot A

Which type of RAM module has 168 pins?

- SIMMs

- DIMMs

- RIMMs

- None

Which type of RAM is used mainly for cache memory?

- SRAM

- DRAM

- EDO RAM

- SDRAM

What is stored in RAM?

- Currently running programs

- Programs that aren’t running

- Nothing is stored in RAM

- Hardware information

Which of the following statements regarding master/slave configurations is true?

- You need a slave device to have a master device

- You need a master device to have a slave device

- EIDE systems can have two slaves, but only one master on a single cable

- EIDE systems cannot have two masters or two slaves on a single cable

How many pins on the ribbon cable are used to connect an IDE drive?

- 36

- 40

- 60

- 80

Where do you place the master hard drive on a standard, non-cable select EIDE cable?

- At the end of the cable

- In the middle of the cable

- It doesn’t matter

- EIDE cable are for floppy drives

What CMOS function resets all the PnP information for the system?

- Legacy

- Reset Configuration Data

- Clear CMOS

- Advanced system

What is the main difference between AT and ATX power supplies?

- ATX power supplies require 240VAC input.

- The ATX power supply is controlled by a software switch on the system board.

- ATX power supplies deliver more power to the system.

- AT power supplies blow air onto the system board rather than out through the back of the system.

Which type of storage is volatile?

- RAM

- CD-ROM

- Disk

- ROM

How many DMA channels does an AT-compatible system have?

- 2

- 4

- 8

- 16

On which I/O address would you find the primary IDE controller in a PC-compatible system?

- 070

- 170

- 1F0

- 3F8

On which I/O address would you find the second IDE controller in a PC-compatible system?

- 070

- 170

- 278

- 378

What does the computer typically use to stop the processes of the microprocessor so that attention can be given to the demands of a particular device?

- ATX

- ATA

- IRQ

- NMI

Which device normally uses IRQ7?

- A laser printer

- A modem

- A floppy disk drive

- A hard disk drive

What distinctive feature enables you to quickly identify an Ultra ATA 66 interface cable?

- The connectors are blue.

- The cable is blue.

- The cable has 40 wires.

- The cable has 80 wires.

What type of device would you expect to find in an AGP slot?

- AMR card

- SCSI card

- Video card

- Network card

What is the first step in checking out electrical equipment that appears dead?

- Check the power supply connection to the motherboard.

- Check the motherboard battery.

- Check to see that the power cord is plugged in.

- Check to see the power light is on.

___________ is the gradual deterioration of the electrical connection between the pins of an IC and its socket.

- Rust

- Degradation

- Chip creep

- Tarnish

What is the order of execution for the files involved in the MS-DOS boot process?

A. IO.SYS, MSDOS.SYS, COMMAND.COM, CONFIG.SYS, AUTOEXEC.BAT

B. IO.SYS, MSDOS.SYS, CONFIG.SYS, AUTOEXEC.BAT, COMMAND.COM

C. IO.SYS, MSDOS.SYS, CONFIG.SYS, COMMAND.COM, AUTOEXEC.BAT

D. MSDOS.SYS, IO.SYS, CONFIG.SYS, COMMAND.COM, AUTOEXEC.BAT

Which file loads the Windows 9x operating system into the system?

A. START.EXE

B. IO.SYS

C. WIN.COM

D. AUTOEXEC.BAT

What are the names of the files involved in the Windows NT boot process, and in what order are they run?

A. BOOT.INI, NTLDR, NTOSKRNL.EXE, NTDETECT.COM

B. NTLDR, BOOT.INI, NTOSKRNL.EXE, NTDETECT.COM

C. BOOT.INI, NTLDR, NTDETECT.COM, NTOSKRNL.EXE

D. NTLDR, BOOT.INI, NTDETECT.COM, NTOSKRNL.EXE, HAL.DLL

A device is using IRQ 2.  Which other IRQ should NOT be used by any other device?

A. IRQ 3

B. IRQ 4

C. IRQ 8

D. IRQ 9

Which of these types of files cannot be executed from the command-line prompt?

A. .SYS

B. .COM

C. .EXE

D. .BAT

Which DOS wildcard characters tell the software to perform the designated command on any file found on the disk using any filename and extension?

A. # . #

B. * . *

C. ? . ?

D. + . +

What are the primary characteristics of an extended partition? (Select all that apply)

A. It is the first partition on the drive.

B. It may be divided into 23 logical drives.

C. It cannot be deleted if logical drives have been defined within it.

D. It is the logical drive from which the system will boot.

What is the smallest unit of storage on a disk that can be manipulated by the operating system?

A. pixel

B. bit

C. cluster

D. byte

What is the size of a sector in an IBM/PC-compatible disk?

A. 512 bytes

B. 8 bytes

C. 16 bytes

D. 1,024 bytes

Which of the following characters cannot be used in an MS-DOS filename? (Select two answers)

- ?

- &.

- >

- #

What are the primary characteristics of an active partition?

A. It is the first partition on the drive.

B. It may be divided into 23 logical drives.

C. It cannot be deleted if logical drives have been defined within it.

D. It is the logical drive from which the system will boot.

What command can be used to recover, and optionally delete, lost file segments in Windows 9x?

- DISKSCAN /F

- CHECKDISK /F

- SCANDISK /F

- CHKDSK /F

What does the exclamation point (!) inside a yellow circle mean when used by the Windows Device Manager?

- The device has been disabled due to a user-selection conflict.

- It indicates expandable and collapsible information branches.

- The device is experiencing a direct hardware conflict with another device.

- The device is not installed properly.

What command can you use from the command line to show a listing of all the Windows 9x bootup switches?

- WIN ?

- WIN /?

- WIN /HELP

- WIN /H

In a Windows 9x system, which file would you access to correct a HIMEM.SYS error reported during startup?

- COMMAND.COM

- AUTOEXEC.BAT

- CONFIG.SYS

- MSDOS.SYS

The purpose of a device driver is to:

- tell the operating system how to interface with an external device.

- provide more useful memory by moving device control data to extended memory.

- improve performance of installed devices by optimising access patterns.

- modify application programs to work correctly with devices attached to the system.

____________________ interprets input from the command-line.

- IO.SYS

- MSDOS.SYS

- COMMAND.COM

- BIO.COM

A customer cannot find the file QUESTIONPOOL.DOC while searching in MS-DOS. The customer should:

- look for an abbreviated name for the file, such as QUESTI~1.DOC.

- assume that the file has been deleted.

- look for the file in Windows 95 instead.

- look for an abbreviated name for the file, such as QUESTION.DOC

How do upper memory and high memory differ?

- There is no difference.

- Upper memory is any memory above 1MB.

- High memory is the memory between 640kB and 1MB.

- High memory is the first 64kB area of memory above 1MB.

Memory from 0kB to 640kB is called ___________ memory.

- enhanced

- conventional

- expanded

- extended

Jim has just installed a new hard drive in his Windows 98 PC, and now needs to prepare it. In what order should he do the following tasks?

- Format, FDISK, reboot

- FDISK, reboot, format

- Format, reboot, FDISK

- Reboot, format, FDISK

Which of the following is NOT a type of file attribute?

- administrator

- hidden

- system

- read-only

Which of the following is first read by the CPU when a PC is started?

- MBR

- IO.SYS

- IPL

- BIOS

Which file would not be found with this command:  dir we??.e*  ?

- WELL.EXE

- WEIN.E

- WESTY.ERD

- WEON.EX

Which of the following files cannot be edited?

- CONFIG.SYS

- SYSTEM.INI

- AUTOEXEC.BAT

- HIMEM.SYS

Which of the following are created by a partitioning program like FDISK?

- Root directory

- Master boot record

- File allocation tables

- Drive letters

You need to copy everything in a directory from one drive to another. You use the xcopy command. After restarting the machine you notice that the new folder contains only the files from the root of the old folder and did not copy all sub-directories. What command should you have used?

- DISKCOPY

- COPY /S

- XCOPY /32

- XCOPY /S

The command FORMAT.COM /S will cause which of the following? (Select all that apply)

- Format the C: drive

- Format the C: drive in safe mode

- Create a subdirectory on the C: drive

- Make the C: drive bootable

Which of the following statements accurately describes CONFIG.SYS?  (Select all that apply)

- Your system can run fine without a CONFIG.SYS file

- CONFIG.SYS is known as a BIOS loader

- CONFIG.SYS is a static file. It cannot be modified.

- If you notice an unfamiliar line in CONFIG.SYS, it is best to delete that line.

Of the following, which provides the most common software loading sequence?

- Operating system, BIOS, application

- BIOS, operating system, application

- Operating system, application, BIOS

- BIOS, application, operating system

During bootup, the system displays “Fatal Disk Error”. What is the likely cause?

- A loose CPU

- An incorrect setting in CMOS

- An unconnected floppy drive

- An incorrect setting in ROM BIOS

Which of the following procedures will show you the version of Windows a particular system is running? (Choose all that apply)

- Type VER at the command prompt

- Double click the General icon in Control Panel and click the System tab

- Type SET at the command prompt

- Double click the System icon in Control Panel and click the General tab

Which of the following is not a valid command-line program or instruction?

- FDISK

- CLEAN

- COPY

- ATTRIB

What wildcard character can be user to replace a single character in a search string?

- ?

- %

- “

- *

The command prompt to get rid of a full directory and all it’s contents is ____ .

- DELTREE

- REMDIR

- RD

- REMTREE

When a user deletes a file, empties the recycle bin, what happens to the file?

- Sectors of hard drive are blanked/erased.

- Clusters are flushed.

- The file is moved to C:\Windows\temp.

- Associated entries in the FAT are removed.

Which GUI text editors are used to edit the win.ini and autoexec.bat files? (Choose two)

- SYSPAD

- SYSEDIT

- NOTEPAD

- REGEDT32

- REGEDIT

Where is the DIR command located?

- C:\WINDOWS\SYSTEM

- C:\WINDOWS

- COMMAND.COM

- C:\WINDOWS\COMMAND

What is the command syntax for assigning "Hidden" attribute to file c:\myfile?

- ATTRIB +R C:\MYFILE

- ATTRIB +H C:\MYFILE

- ATTRIB -H C:\MYFILE

- ATTRIB H  C:\MYFILE

Your customer has bought a second hard disk and added it an existing computer. The first hard disk is having one primary partition and one Extended partition. The extended partition is divided into two logical drives D and E. When you install the second hard drive and format it, what would be the drive letter assigned to the first logical drive on the extended partition of first hard disk? Assume that the second hard disk has one primary partition and only one logical drive on the Extended partition.

- "D"

- "E"

- "F"

- "G"

# MULTIPLE CHOICE QUESTIONS (1 mark each)

SCSI chains require a terminating _______________.

A. capacitor

B. diode

C. resistor

D. coil

Which of the following is NOT a valid SCSI connector?

A. 15-pin Centronics

B. 50-pin Centronics

C. 25-pin DB

D. 68-pin connector

A 15-pin female DB connector on the back of a sound card could be which of the following?

A. SCSI port

B. Parallel port

C. Joystick port

D. Serial modem port

A device is using IRQ 2.  Which other IRQ should NOT be used by any other device?

A. IRQ 3

B. IRQ 4

C. IRQ 8

D. IRQ 9

You need to configure a device’s IRQ. What is IRQ6 mostly used for?

A. Floppy controller

B. COM2

C. COM3

D. COM4

E. LPT1 or LPT2

John is told his printer port is using a DMA channel. He immediately knows that his parallel port is using _____ mode.

A. Centronics

B. EPP

C. ECP

D. Nibble

Which high-speed computer port is commonly used by streaming video devices like digital cameras?

A. USB

B. Parallel

C. Firewire

D. Serial

What function does IRQ0 play in an PC-compatible system?

A. It drives the system’s DRAM Refresh signal

B. It drives the system’s time-of-day clock

C. It drives the system’s FDD interrupt

D. It drives the system’s keyboard interrupt

Which of the following uses a serial bus architecture? (Select all that apply)

A. USB

B. Firewire

C. IEEE 1284

D. IEEE 1449

To view all of the I/O addresses, IRQs, and DMA settings in Windows XP, you would:

- go to Start Menu, select Open, then select Properties.

- go the Startup menu, and choose View All System Properties.

- go to  Control Panel, select System, then click Computer.

- right click on My Computer, then click Properties, Hardware, Device Manager

Which of these connectors can you use to connect a keyboard to a PC? (Select all that apply)

- Mini-DIN

- 9-pin serial

- 25-pin parallel

- USB

How is data transferred in serial communications?

- 10 bits at a time

- 1 bit at a time

- 1 byte at a time

- 16 bits at a time

How is data transferred in parallel communications? (Select all that apply)

- 8 bits at a time

- 1 byte at a time

- 1 bit at a time

- 8 bytes at a time

To what does a Type A USB connector connect?

- To a USB port on the PC

- To a USB device

- To a serial port on the back of your PC

- To a USB modem

Which of the following statements is true of USB?

- USB enables 12 Mbps data transfers

- USB is 16-bits wide

- USB slots are small and white

- USB slots are backwardly compatible and can support 8-bit or 16-bit ISA cards.

Which of the following devices can take advantage of EPP? (Select all that apply)

- Scanners

- LAN adapters

- Disk drives

- Printers

You need to configure a device IRQ. Given the following IRQs, which ones are mostly available for device assignment? (Choose two)

- IRQs 1, 2 & 3

- IRQs 2, 4 & 6

- IRQs 3, 5 & 7

- IRQs 4, 6 & 8

- IRQs 10, 11 & 12

The resolution for a video adapter operating in native VGA mode is:

- 350 x 468

- 640 x 480

- 800 x 600

- 1024 x 768

PC-compatibles use a ___________ connector for the VGA video function.

- 9-pin, female D-shell

- 15-pin, male D-shell

- 15-pin, female D-shell

- 25-pin, male D-shell

When you are selecting a mouse for a particular computer system, what is the most important consideration?

- The type of drivers that come with the mouse

- The length of the mouse cord

- The type of connector the mouse is equipped with

- The number of buttons the mouse has

Which standard ATX ports are often confused with each other?

- The serial port and the VGA/SVGA port

- The docking port and the parallel ports

- The NIC card and modem connector ports

- The mouse and keyboard connections

Which motherboard form factor uses a 5-pin DIN connector for keyboard connections? (Choose two)

- ATA

- ATX

- AT

- XT

How many devices can be connected to a single Universal Serial Bus (USB) host?

- 63

- 127

- 255

- 511

Which new serial bus is the fastest?

- IrDA

- ECP

- IEEE-1394

- USB

Which IRQ channel and I/O address are typically used for standard serial ports?

- IRQ3, 2F8

- IRQ6, 3F0

- IRQ7, 378

- IRQ15,170

What type of port uses a DB-25F connector?

- VGA port

- game port

- printer port

- COM2 port

Which IRQ channel and I/O address are typically used for standard parallel ports?

- IRQ3, 3F8

- IRQ6, 3F0

- IRQ7, 378

- IRQ15, 370

What type of problem causes a “Press F1 to continue” error message to be displayed?

- A floppy drive is defective.

- The operating system is missing from the hard drive.

- The system needs to be reconfigured.

- The video display is defective.

What are the voltage levels commonly found in a CRT?

- 25 volts

- 250 volts

- 25,000 volts

- 250,000 volts

When your computer boots up, you want it to search for a boot sector on a floppy, a CD-ROM drive, and then the hard disk drive. What boot sequence should you set in CMOS?

- A, C, SCSI

- A, C, CD-ROM

- CD-ROM, A, C

- A, CD-ROM, C

Which specification employs DMA operations to provide the highest data throughput for a parallel port?

- PPP

- SPP

- EPP

- ECP

Where is the PnP information stored in the BIOS?

- The RTC module

- The ESCD area

- The PnP registry

- The Device Manager

What is the major difference between EPP and ECP operation of the parallel port?

- DMA mode

- bidirectional

- 16-bit

- serial

A connector that has 15 pins in two rows is which type of port?

- Monitor port

- Modem port

- Serial port

- Game port

A parallel printer cable has a different connector on each end. Name the two types of connectors on a parallel printer cable. (Choose two)

- DB 25-pin male connector

- DB 9-pin connector

- An RS-232 compliant cable

- 36-pin Centronics connector

Which default address does the primary IDE controller use?

- 110H

- 470H

- 1F0H

- 300H

- 240H

Which of the following describe differences between an ISA expansion slot and a PCI expansion slot? (Select all that apply)

- PCI slots are shorter.

- PCI slots are 64-bits.

- PCI slots are primarily used for video graphics adapters.

- PCI slots are longer.

You are attempting to connect a USB mouse, a USB keyboard, and USB digital camera to your new USB 2.0 compatible laptop system. How many system resources (IRQs) will you need?

- One

- Three

- 1,024

- You cannot connect USB 2.0 device to a laptop.

A good example of firmware is:

- CONFIG.SYS

- Windows 95

- DOS

- CMOS ROM

If a modem listed in the Device Manager indicates a resource conflict, what action should be taken to clear up the problem?

- Run MSD.EXE

- Run MEM.EXE

- Click the Resources tab and check for conflicts

- Change the IRQ setting for the device